# Accessibility survey questionnaire

Tiếp Cận project · Version v2-qcvn2024 · updated 2026-10-29

Thresholds refer to QCVN 10:2024/BXD (Circular 06/2024/TT-BXD, in force from 1 Feb 2025; it replaced the 2014 version). The PARTIALLY level is more lenient than the standard (door 80-89 cm, threshold up to 2 cm): it is a project criterion, not a pass/fail finding against QCVN.

## A. General information

| Place name |  | Type of place |  |
| --- | --- | --- | --- |
| Address |  |  |  |
| City / province |  | GPS (latitude, longitude) |  |
| Date and time of survey |  | Surveyor |  |

GPS: take it only when standing right at the main entrance, using the "Get GPS location" button of the online form (error at most 30 m); do not type it in by hand. The paper questionnaire is for notes on site, to be entered into the online form afterwards.

How to use: for each question tick Yes / No / Not sure; for measurements write the number. Questions marked (take an illustrative photo) should be backed by a photo. The questionnaire follows the photo positions: B Overview, C Entrance, D Main door, E Lift, F Toilet. Photos: 3 required positions (Overview taken in landscape 16:9, Entrance, Main door) and 2 optional positions (Lift, Toilet); at most 5 photos per position, at least 5 in total. Do not show anyone's face without their consent.

### A2. Survey together with a wheelchair user (priority)

| No. | Question | Answer |
| --- | --- | --- |
| 1 | Is a person who uses a wheelchair surveying together with you? | Yes No Not sure |
| 2 | Has that person agreed to be photographed and to the photos being used by the project? (only if a wheelchair user is with you) | Yes No Not sure |
| 3 | Is there at least 1 photo showing that person (with their consent) at the entrance, door or toilet? Use it as a public photo only if they agree to publication, otherwise put it in the Extra photos (take an illustrative photo) (only if a wheelchair user is with you) | Yes No Not sure |

### B. Overview (getting there, inside, assistance)

| No. | Question | Answer |
| --- | --- | --- |
| 4 | Is the pavement in front of the door usable by wheelchair (flat, wide enough, not blocked by motorbikes or stalls)? (take an illustrative photo) | Yes No Not sure |
| 5 | Is there disabled parking or a drop-off point close to the door? | Yes No Not sure |
| 6 | Is the way inside wide enough for a wheelchair to turn around (clear space of at least about 1.5 m)? | Yes No Not sure |
| 7 | Does the place have more than one floor? | Yes No Not sure |
| 8 | Does the reception / cashier counter have a low section or knee space so a wheelchair user can use it? QCVN 10:2024/BXD, clause 2.5.1.3 (counter top ≤ 800 mm; knee space ≥ 650 mm) | Yes No Not sure |
| 9 | Do the staff know how to help / are they willing to help when asked? | Yes No Not sure |
| 10 | Is there a sign showing the accessible route? QCVN 10:2024/BXD, clause 2.2.7 | Yes No Not sure |

### C. Entrance

| No. | Question | Answer |
| --- | --- | --- |
| 11 | Number of steps at the main entrance (0 if none) (take an illustrative photo) QCVN 10:2024/BXD, clause 2.2.2; 2.2.5 | ……………… steps |
| 12 | Height of the highest step QCVN 10:2024/BXD, clause 2.2.5 (≤ 150 mm) (only if there are steps) | ……………… cm |
| 13 | Is there a ramp instead of the steps? (take an illustrative photo) QCVN 10:2024/BXD, clause 2.2.2; 2.2.3 (only if there are steps) | Yes No Not sure |
| 14 | Slope of the ramp QCVN 10:2024/BXD, clause 2.2.3 (≤ 1/12) (only if there is a ramp) | No steeper than 1/12 Steeper than 1/12 |
| 15 | Width of the ramp QCVN 10:2024/BXD, clause 2.2.3 (≥ 1.200 mm) (only if there is a ramp) | ……………… cm |
| 16 | If the main entrance is not accessible: is there another entrance? (only if there are steps and no ramp) | Yes No Not sure |
| 17 | Where is the other entrance? (only if there is another entrance) | ………………………………… |

### D. Main door

| No. | Question | Answer |
| --- | --- | --- |
| 18 | Clear width of the main door (measured from the inner edge, with the door fully open) (take an illustrative photo) QCVN 10:2024/BXD, clause 2.3.1 (entrance door ≥ 900 mm; inner rooms ≥ 800 mm) | ……………… cm |
| 19 | Door type QCVN 10:2024/BXD, clause 2.2.6 (no revolving door) | Automatic Easy to push Heavy, hard to open |
| 20 | Threshold at the main entrance QCVN 10:2024/BXD, clause 2.2.6 (no threshold) | No threshold Yes, up to 2 cm high Yes, over 2 cm high |

### E. Lift (if any)

| No. | Question | Answer |
| --- | --- | --- |
| 21 | Is there a lift to the floors? QCVN 10:2024/BXD, clause 2.4 (only if more than one floor) | Yes No Not sure |
| 22 | Width of the lift door QCVN 10:2024/BXD, clause 2.4.1 (≥ 900 mm) (only if there is a lift) | ……………… cm |
| 23 | Height of the lift control panel inside the car: HIGHEST button (floor of the lift to the centre of the button). To know whether a wheelchair user can reach the buttons (take an illustrative photo) QCVN 10:2024/BXD, clause 2.4.4 (highest button ≤ 1200 mm) (only if there is a lift) | ……………… cm |
| 24 | Height of the lift control panel inside the car: LOWEST button (floor of the lift to the centre of the button) (only if there is a lift) | ……………… cm |
| 25 | Do the lift buttons have raised characters (Braille or tactile symbols) for blind users? QCVN 10:2024/BXD, clause 2.4.4 (only if there is a lift) | Yes No Not sure |

### F. Toilet

| No. | Question | Answer |
| --- | --- | --- |
| 26 | Is there a toilet for people with disabilities? (take an illustrative photo) QCVN 10:2024/BXD, clause 2.5.5.1 | Yes No Not sure |
| 27 | If not: does the ordinary toilet have a step or a narrow door? (short description) (only if there is no accessible toilet) | ………………………………… |
| 28 | Width of the accessible toilet door QCVN 10:2024/BXD, clause 2.5.5.5 (≥ 800 mm) (only if there is an accessible toilet) | ……………… cm |
| 29 | Are there grab bars? QCVN 10:2024/BXD, clause 2.5.5.7 (only if there is an accessible toilet) | Yes No Not sure |
| 30 | Is there enough room to turn a wheelchair? QCVN 10:2024/BXD, clause 2.5.5.4 (≥ 1.400 × 1.400 mm) (only if there is an accessible toilet) | Yes No Not sure |
| 31 | Is the toilet seat at a suitable height? QCVN 10:2024/BXD, clause 2.5.5.6 (toilet seat ≤ 450 mm) (only if there is an accessible toilet) | Yes No Not sure |

### Extra section for hotels / guesthouses

| No. | Question | Answer |
| --- | --- | --- |
| 32 | Is there an accessible room? | Yes No Not sure |
| 33 | Which floor is the room on, and is there a lift to it? (only if there is an accessible room) | ………………………………… |
| 34 | Width of the room door and the bathroom door inside the room (only if there is an accessible room) | ………………………………… |
| 35 | Does the bathroom have a shower seat / bath chair and grab bars? (take an illustrative photo) (only if there is an accessible room) | Yes No Not sure |
| 36 | Height of the bed (only if there is an accessible room) | ……………… cm |
| 37 | Can the switches, wardrobe and hanging space be reached from a wheelchair? (only if there is an accessible room) | Yes No Not sure |
| 38 | Are the breakfast area and shared areas accessible? | Yes No Not sure |
| 39 | Is there a policy for booking accessible rooms / notifying the hotel in advance? | Yes No Not sure |

### Extra section for restaurants / eateries

| No. | Question | Answer |
| --- | --- | --- |
| 40 | Is there a table where a wheelchair can fit (tables not fixed close together)? | Yes No Not sure |
| 41 | Height under the table | ……………… cm |
| 42 | Is there a large-print or tactile menu, or staff who read the menu? | Yes No Not sure |
| 43 | Is the toilet on the same floor as the dining room? | Yes No Not sure |

### Extra section for museums, heritage sites and attractions

| No. | Question | Answer |
| --- | --- | --- |
| 44 | Is the ticket counter accessible (low counter, staff help)? | Yes No Not sure |
| 45 | Do people with disabilities and their companions get free or discounted tickets? | Yes No Not sure |
| 46 | Are wheelchairs available to borrow? (take an illustrative photo) | Yes No Not sure |
| 47 | Does the visiting route have ramps / lifts linking the areas? State clearly which areas are NOT accessible (and why, for example conservation) | ………………………………… |
| 48 | Surface of the courtyard / paths | Flat, firm Gravel / rough stone Grass / soil Uneven bricks |
| 49 | Are there seats and shade along the route? | Yes No Not sure |

### Extra section for train stations, bus stations, airports and bus shelters

| No. | Question | Answer |
| --- | --- | --- |
| 50 | Does the ticket / check-in counter have a low section and staff to help? | Yes No Not sure |
| 51 | Are staff available to help passengers on and off? (note how to book if needed) | Yes No Not sure |
| 52 | Stations / bus bays: gap between the platform / bay and the vehicle floor (cm) | ……………… cm |
| 53 | Height difference between the platform / bay and the vehicle floor (cm) | ……………… cm |
| 54 | Does the vehicle have a low floor / ramp / lift / wheelchair space? | Yes No Not sure |
| 55 | Bus shelter: flat floor, enough room for a wheelchair (>= 1.2 x 0.8 m), lowered kerb? | Yes No Not sure |
| 56 | Are announcements given by sight and sound, and is the timetable readable? | Yes No Not sure |

### For airports / train stations: assistance policy for people with disabilities

| No. | Question | Answer |
| --- | --- | --- |
| 57 | Do people with disabilities have to register / notify in advance to get assistance? | Not needed Advance notice advised (not required) Advance notice required Not sure |
| 58 | How and by when to register (phone, email, how long in advance) (only if notice is advised or required) | ………………………………… |
| 59 | Is there a ticket discount for people with disabilities? | Yes No Not sure |
| 60 | Amount of discount and documents required (only if there is a discount) | ………………………………… |
| 61 | Is there a lift / platform lift / device to help passengers board the aircraft cabin or the train? (take an illustrative photo) | Yes No Not sure |
| 62 | Is there a lift / device to help from the aircraft to the terminal (or from the train to the platform)? | Yes No Not sure |
| 63 | Type of equipment (for example ambulift, boarding bridge, special wheelchair, platform lift) | ………………………………… |
| 64 | Is an extra service fee charged for people with disabilities? | Yes No Not sure |
| 65 | Fee amount and what service it covers (only if a fee is charged) | ………………………………… |

### Extra section for offices, organisation headquarters and public administrative service centres

| No. | Question | Answer |
| --- | --- | --- |
| 66 | Is there a low service counter (or part of a counter) for wheelchair users? | Yes No Not sure |
| 67 | Height of the lowest service counter surface (only if ag_counter=Có) | ……………… cm |
| 68 | Are the counters and rooms that serve the public on the ground floor, or reachable by lift / ramp? | Yes No Not sure |
| 69 | Name the counters or rooms that CANNOT be reached (only if ag_floor=Không) | ………………………………… |
| 70 | Do the queue ticket and call system show on screen and announce by voice? | Yes No Not sure |
| 71 | Is there staff on site to guide and help people with disabilities? | Yes No Not sure |
| 72 | Is there a priority counter or lane for people with disabilities? | Yes No Not sure |
| 73 | Are there waiting seats and a waiting space for wheelchairs? | Yes No Not sure |
| 74 | Is there support for deaf people (sign language, text or video)? | Yes No Not sure |
| 75 | Can people with disabilities file online or receive results at home? | Yes No Not sure |

### Extra section for hospitals and health facilities

| No. | Question | Answer |
| --- | --- | --- |
| 76 | Is there a pick-up and drop-off point right at the door (ambulance, taxi), with a roof? | Yes No Not sure |
| 77 | Does the registration and payment counter have a low section? | Yes No Not sure |
| 78 | Are the clinics, laboratory and imaging departments on the ground floor, or reachable by lift / ramp? | Yes No Not sure |
| 79 | Name the departments or areas that CANNOT be reached (only if hc_floors=Không) | ………………………………… |
| 80 | Width of the examination room door | ……………… cm |
| 81 | Is the examination bed or table height-adjustable, or is there a patient transfer aid? | Yes No Not sure |
| 82 | Does the imaging room (X-ray, CT, ultrasound) help transfer people from a wheelchair to the table? | Yes No Not sure |
| 83 | Inpatient wards: can a wheelchair use the room door, the way through and the toilet in the room? | Yes No Not sure |
| 84 | Does the emergency department have a level or ramped entrance with a wide enough door? | Yes No Not sure |
| 85 | Are wheelchairs or stretchers available to borrow at the door? (take an illustrative photo) | Yes No Not sure |
| 86 | Are signs large and high-contrast, with raised characters or guiding strips for blind users? | Yes No Not sure |
| 87 | Is there support for deaf patients (sign language, writing board, text)? | Yes No Not sure |

### Extra section for facilities and centres of people with disabilities

| No. | Question | Answer |
| --- | --- | --- |
| 88 | Main people served (for example blind, deaf, physical, intellectual, autism) | ………………………………… |
| 89 | Main services and activities (for example vocational training, rehabilitation, care, counselling) | ………………………………… |
| 90 | Do people with disabilities work in or help run the place? | Yes No Not sure |
| 91 | Can a wheelchair use the activity, training and living rooms (if any)? | Yes No Not sure |
| 92 | Are signs large and high-contrast, with raised characters or guiding strips for blind users? | Yes No Not sure |
| 93 | Do bells and alarms have flashing lights or vibration for deaf people? | Yes No Not sure |
| 94 | Is there a sign language interpreter? | Yes No Not sure |
| 95 | Is there a quiet room or a way to reduce stimulation for autistic or sensory-sensitive people? | Yes No Not sure |
| 96 | Can people borrow or repair wheelchairs and assistive devices? | Yes No Not sure |
| 97 | Is there a pick-up vehicle that a wheelchair can use? | Yes No Not sure |
| 98 | Do visitors have to book ahead? | Not needed Advance notice advised (not required) Advance notice required Not sure |

### Extra section for universities, inclusive schools and special schools

| No. | Question | Answer |
| --- | --- | --- |
| 99 | Types of disability the school accepts or teaches (if known) | ………………………………… |
| 100 | Does it accept students with disabilities and offer support (fee reduction, special admission)? | Yes No Not sure |
| 101 | Number of students with disabilities now studying (if known) | ……………… people |
| 102 | Is there a unit or person in charge of inclusive-education support? | Yes No Not sure |
| 103 | Can a wheelchair enter classrooms, lecture halls and laboratories (door and way in)? | Yes No Not sure |
| 104 | Do the upper floors have a lift, or are classes with students with disabilities held on the ground floor? | Yes No Not sure |
| 105 | Is there a space for a wheelchair or adjustable desks in the classroom? | Yes No Not sure |
| 106 | Are the yard and paths between buildings level, with ramps instead of steps? | Yes No Not sure |
| 107 | Is the canteen accessible? | Yes No Not sure |
| 108 | Are the library and computer rooms accessible, with braille, audio materials or screen readers? | Yes No Not sure |
| 109 | Does the dormitory or boarding area have accessible rooms? | Yes No Not sure |
| 110 | Is there sign language interpreting or captioning for deaf students? | Yes No Not sure |
| 111 | Is there a pick-up vehicle that a wheelchair can use? | Yes No Not sure |

## G. Overall accessibility level

The surveyor chooses one level and writes a one-sentence comment:

FULLY ACCESSIBLE: can be entered, can be moved around inside, and the toilet can be used without anyone's help.

PARTLY ACCESSIBLE: can be entered but one condition is missing (for example the toilet cannot be used, or someone has to lift the wheelchair over a step).

NOT ACCESSIBLE: cannot be entered independently.

Comment: ……………………………………………………………………………………………………………

Hard rule: an entrance counts as fully compliant only with no steps or a ramp no steeper than 1/12 and at least 1,200 mm wide (QCVN 2.2.3), an entrance door of at least 900 mm (2.3.1) and no threshold (2.2.6); a door of 800-899 mm or a threshold up to 2 cm gives at most PARTLY; a door under 800 mm or a threshold over 2 cm gives NOT. A place is rated FULLY ACCESSIBLE only when ALL THREE conditions are met: the entrance, the way inside, and the toilet. If one of the three fails the maximum is PARTLY ACCESSIBLE. This rule is public.

### H. Blue ribbon

| No. | Question | Answer |
| --- | --- | --- |
| 112 | Does this place employ people with disabilities? | Yes No Not sure |
| 113 | Level at which people with disabilities are employed (choose one or more) (only if the place employs people with disabilities) | Management level Staff in suitable departments Other |
| 114 | Other level (please describe) (only if "Other" is ticked) | ………………………………… |
| 115 | Type of recruitment (only if the place employs people with disabilities) | Regular recruitment Only when a position is vacant |
| 116 | How many employees with disabilities does the place have now? (only if the place employs people with disabilities) | ……………… people |
| 117 | Does the place regularly welcome customers with disabilities? | Yes No Not sure |
| 118 | Is the place regularly chosen to hold programmes or events for people with disabilities? | Yes No Not sure |

Surveyor's declaration: the measurements and photos are genuine; I have not accepted money or gifts to change the result. Signature: ……………………